Assembling the Mark4-8 V2 Partizan Edition Frame sku 6422

Equipment:

| N | Photo | Quantity |
| --- | --- | --- |
| 1 |  | M2x5mm -2 pcs. |
| 2 |  | M3x8mm -12 pcs. |
| 3 |  | M3x10mm -4 pcs |
| 4 |  | M3x12mm -8 pcs. |
| 5 |  | M3x14mm -4 pcs |
| 6 |  | M3x16mm -4 pcs |
| 7 |  | M3x25mm -4 pcs |
| 8 |  | 25mm -8 pcs. |
| 9 |  | M3 nut -4pcs |
| 10 |  | M3 nut with rubber band -12pcs |
| 11 |  | Beam Mounting Board -1pc |
| 12 |  | Bottom board -1pc |
| 13 |  | Top board -1pc |
| 14 |  | Rays -4pcs |
| 15 |  | Jumper to the beams -2pc |
| 16 |  | Camera mount -2pc |
| 17 |  | Plastic antenna holder VTX -1pc |
| 18 |  | Plastic RX antenna holder -1pc |
| 19 |  | Battery rubber band -1pc |
| 20 |  | Strap for securing the battery -2pc |

CONSTRUCTION:

| Frame assembly |  |  |
| --- | --- | --- |
|  |  | Sorting out the largest bolts. Secure the rays. Details: 5+11+12+14 Use M3x14mm bolts to fasten the beams between the bottom board and the board to secure the beams Picture 1: Bottom view. The arrows indicate the position of the part to be observed Picture 2: Top view |
|  |  | Detail 4- bolt M3x16mm -4 pcs Detail 8 - rack M3x25mm -4 pcs Align the beams so that the bolt holes converge. Bolt the beams to the posts. |
|  |  | Detail 2- M3x8mm -4 Detail 8 - rack M3x25mm -4 pcs 1/ Install the remaining 4 posts on the lower enclosure board. Insert the bolt caps into the recesses. |
|  |  | Detail 4- Bolt M3x12mm -8pcs Detail 15-Crosspiece -2p Detail 10 - M3 nut with rubber band -8pcs Jumpers are placed on top of the beams. 1 / Set the Jumpers on the beams. 2/ tighten with nuts with locking rubber band |
|  |  | Detail 7 - Bolt M3x25mm -4 pcs Screw in and tighten the bolts to secure the stack. |
|  |  | Tighten all Bolts, Stays, Nuts. Hold the bolts with a screwdriver and tighten the nut with a 5.5 wrench. Holding the rack firmly with pliers, tighten its bolt. |